# ECE 280L: Signals & Systems

Revised Arduino-Based Laboratory Schedule

Hardware-Integrated, LLM-Resistant Lab Curriculum

| Wk | Lecture Topics | Lab Title | Lab Description | LLM-Proof Elements |
| --- | --- | --- | --- | --- |
| 1 | Intro to Signals & Systems; CT and DT Signals | Warm-Up: Arduino Signal Generator | Build Arduino function generator (sine, square, triangle via PWM/DAC). Capture output with oscilloscope. Simultaneously acquire data into MATLAB via Serial and plot. Compare scope capture to MATLAB plot. Students learn the Arduino→MATLAB data pipeline used all semester. | Unique freq/amplitude combo per student. Scope photo with student name on screen + matching MATLAB figure required. Hand-drawn signal sketches from live measurements. |
| 2 | Signal Transformations; Exponentials & Sinusoids; Unit Impulse/Step | Signal Transformations in Hardware | Arduino outputs a base signal; students apply time shift, scaling, and reversal using potentiometers and code. Acquire both original and transformed signals into MATLAB. Use MATLAB to verify transformations match theory. Compare hardware output vs. circshift/fliplr results. | Each student gets a unique random-seed base signal from TA. Report requires annotated scope captures + MATLAB overlay of predicted vs. measured with hand calculations shown. |
| 3 | CT/DT Systems; Basic System Properties | System Properties Explorer | Build 3 circuits (RC filter, diode clipper, comparator) with Arduino as source. Apply test inputs to check linearity, TI, causality, stability. Acquire input/output pairs into MATLAB. Use MATLAB scripts to numerically verify superposition and time-invariance from captured data. | Physical circuit photo with student ID. Unique test inputs per student. Hand-written property truth tables with scope evidence. MATLAB verification code must use their acquired data, not synthetic. |
| 4 | LTI Systems; Convolution Sum & Integral | Convolution Machine | Enter h[n] via DIP switches, x[n] via pushbuttons. Arduino computes y[n] and displays on LED bar. Stream all three sequences to MATLAB via Serial. In MATLAB, verify using conv() and create stem plots. Visualize flip-shift-multiply-sum animation in MATLAB. | Unique h[n]/x[n] per pair (TA-assigned). Must show hand flip-shift-multiply-sum for each output sample. Video of LED output + matching MATLAB stem plot required. |
| 5 | Properties of LTI Systems; Causal LTI / Diff Eqs; Singularity Functions | RC/RL Circuit Impulse & Step Response | Build RC and RL circuits with unique component values. Arduino generates narrow pulse (≈impulse) and step. Sample output with Arduino ADC at high rate, stream to MATLAB. In MATLAB, overlay measured response with theoretical exp(-t/tau). Compute time constant from data using curve fitting. | Each bench has different R, C/L values (physically labeled, not shared digitally). Hand-derive transfer function, predict response, then compare to MATLAB fit. Component values verified by TA. |
| 6 | Correlation; Intro to Fourier Series | Correlation-Based Distance Sensor | Build ultrasonic rangefinder: Arduino transmits chirp, records echo via ADC. Stream raw data to MATLAB. Compute cross-correlation in MATLAB using xcorr(). Find peak delay, calculate distance. Also implement correlation on Arduino and compare. Calibrate against ruler at TA-assigned distances. | 5+ unique TA-assigned distances. Must plot raw correlation output in MATLAB for at least 2 distances and hand-annotate peaks. Explain discrepancies between Arduino-computed and MATLAB-computed results. |
| 7 | Fourier Series & LTI; Filtering; TEST 1 | Fourier Series Synthesizer | Arduino synthesizes periodic waveforms by summing harmonics through DAC. Add harmonics one at a time, output to speaker + scope. Stream partial sums to MATLAB for spectral verification using fft(). Demonstrate Gibbs phenomenon visually in MATLAB. | Each student assigned different target waveform (unique duty cycle or shape). Hand-compute Fourier coefficients. MATLAB FFT of hardware output must match predicted spectrum. Audio recording of each partial sum required. |
| 8 | Properties of CT Fourier Series; CT Filters; Parseval's | Analog Filter Bank Spectrum Analyzer | Build 3–4 op-amp bandpass filters at different center frequencies. Arduino generates mystery periodic signal (TA-programmed). Measure each filter output amplitude with Arduino ADC. Stream to MATLAB and reconstruct approximate spectrum. Compare to MATLAB fft() of raw signal. | TA loads unique mystery signal per team. Physical filter circuit photographed. Must identify signal from hardware filter measurements before using MATLAB FFT to confirm. Parseval's check: compare time-domain energy to frequency-domain energy in MATLAB. |
| 9 | CT Fourier Transform; Properties; Convolution Property | Real-Time FIR Filter on Arduino | Design FIR filter coefficients in MATLAB using fir1() with TA-assigned cutoff. Deploy filter to Arduino via Simulink Support Package for Arduino. Process live microphone audio in real time. Measure frequency response by sweeping input frequency, logging output in MATLAB. | Unique cutoff frequencies per student. Must show hand-designed windowed-sinc coefficients alongside MATLAB fir1() output. Frequency response measured point-by-point from hardware — scope captures at each test frequency. Simulink model screenshot required. |
| 10 | Sampling Theorem; Reconstruction; Aliasing | Sampling & Aliasing Demonstrator | Build sample-and-hold circuit (CD4066 analog switch + capacitor) controlled by Arduino timer. Sample known-frequency signals at various rates. Reconstruct with RC lowpass. Acquire sampled and reconstructed signals into MATLAB. Use Simulink to model ideal vs. real reconstruction and compare. | Students predict alias frequencies by hand before measuring. TA assigns unique signal frequencies. Scope photos of reconstructed waveforms at each rate. Simulink model of sampling/reconstruction must match hardware observations. |
| 11 | Fourier Applications; Filtering; Comm Systems Intro; AM | AM Radio Transmitter | Build amplitude modulator with Arduino carrier + AD633 analog multiplier. Modulate audio onto carrier. Observe DSB-SC and DSB on scope. Stream modulated signal to MATLAB, verify sideband structure with fft(). Use Simulink AM modulator block to compare ideal vs. hardware output. | Unique carrier frequency per team. Hand-derive expected spectrum. MATLAB FFT of hardware output must show sidebands at predicted locations. Simulink ideal model vs. hardware overlay required. Measure modulation index from scope envelope. |
| 12 | DSB-SC vs DSB; Envelope Detection; TEST 2 | AM Radio Receiver & Demodulator | Build envelope detector (diode + RC) and synchronous demodulator. Receive signal from neighboring team's transmitter. Acquire demodulated audio in MATLAB. Compare waveform quality of both methods. Use Simulink to model synchronous detection and demonstrate phase-mismatch distortion. | Must receive another team's unique signal (can't fabricate). Scope photos of modulated input + demodulated output side by side. Simulink phase-sweep experiment showing distortion vs. phase error. Hand analysis of distortion sources. |
| 13 | Laplace Transform; ROC; Inverse Laplace | Laplace Transform & Circuit Analysis | Build 2nd-order RLC circuit. Swap resistors to achieve overdamped, underdamped, critically damped responses. Arduino provides step input, ADC captures response. In MATLAB, define tf() transfer function, use step() to generate theoretical response, overlay with measured. Extract poles from measured data using log decrement method. | Component values unique per bench (resistor grab-bag). Hand-derive H(s), find poles, predict damping category before measuring. MATLAB pole-zero map + measured parameter extraction. Hand-drawn pole-zero plot must match. |
| 14 | Laplace Properties; System Functions; Block Diagrams | System Identification Challenge | TA provides sealed circuit modules. Students probe with Arduino test signals (impulse, step, sinusoids). Acquire I/O data in MATLAB. Use tfest() or manual curve-fitting to identify H(s). Build Simulink block diagram of identified system, simulate, and verify against measured data. | Entirely measurement-based with unknown physical systems—impossible to solve without the hardware. Each black box is different. Students present their identification methodology and results live to TA. |

# Design Philosophy

This revised lab curriculum replaces the MATLAB-only approach with a hardware-first pedagogy. Every lab requires students to build physical circuits, take real measurements, and reconcile theory with the messy reality of analog signals. The Arduino serves as both signal source and data acquisition system, while analog circuits provide the signal processing. This approach draws on practices from several programs:

- Georgia Tech ECE 2026 — uses hardware DSP labs with real-time audio processing and physical filter construction

- MIT 6.003 / 6.007 — integrates op-amp filter labs and physical sampling demonstrations alongside computational tools

- University of Illinois ECE 210 — includes AM radio construction as a capstone connecting modulation theory to practice

- Rice University ELEC 241 — employs TI LaunchPad-based labs for real-time convolution and filtering

- Tektronix/Arduino Courseware — oscilloscope-driven signal measurement labs using Arduino as DUT

- Udemy DSP From Ground Up — Arduino-based FFT, convolution kernel, and filter design implementations

# LLM-Proofing Strategy

Every lab incorporates multiple layers of resistance to AI-generated submissions:

- Physical Evidence: Oscilloscope screenshots, breadboard photos with student ID visible, and video demonstrations are required. These cannot be fabricated by an LLM.

- Unique Parameters: Each student/team receives different component values, frequencies, or signal assignments from the TA. Answers are not interchangeable between groups.

- Hand Calculations: Key derivations must be shown in the student’s handwriting (or tablet with stylus), not typed. The derivation must use their specific assigned parameters.

- Live Demonstrations: Several labs require in-person demos to the TA, where students must explain their circuit and answer follow-up questions on the spot.

- Measurement-Theory Comparison: Reports require overlaying measured data with theoretical predictions and explaining discrepancies—something that requires actual hardware interaction.

- Black-Box Challenges: Some labs use sealed unknown circuits that can only be characterized through physical probing, making the problem unsolvable without the hardware.

# Required Equipment Per Bench

- Arduino Uno R4 WiFi (built-in 12-bit DAC) or Arduino Uno R3 + MCP4725 DAC breakout

- Breadboard, jumper wires, resistor/capacitor/inductor assortment kit

- Op-amp ICs (LM358 or TL072), analog multiplier (AD633), comparator (LM311)

- Electret microphone module + audio amplifier module (MAX9814 or similar)

- Small speaker (8Ω) or piezo buzzer

- DIP switches (8-position), pushbuttons, LED bar graph (10-segment)

- Oscilloscope (2-channel minimum), function generator, multimeter

- Ultrasonic transducer pair (for correlation lab)

- Potentiometers (10kΩ linear taper), diodes (1N4148), inductors (10mH, 47mH)

# Grading Rubric Per Lab

Each lab report is graded out of 100 points with the following breakdown:

| Component | Points | Notes |
| --- | --- | --- |
| Pre-lab hand calculations | 15 | Must use assigned parameters |
| Circuit construction & photos | 15 | Student ID visible in photo |
| Measurements & scope captures | 25 | Annotated with cursor measurements |
| Theory vs. measurement comparison | 20 | Overlay plots with error analysis |
| Analysis & discussion of results | 15 | Explain discrepancies |
| Independent exploration (bonus) | 10 | Up to 100 max; extra experiments |

# Software Policy

Students write Arduino C/C++ code for signal generation, data acquisition, and basic DSP operations. Python or MATLAB may be used only for post-processing and plotting of data already acquired from hardware. All signal processing algorithms (convolution, filtering, Fourier coefficient computation) must first be implemented on the Arduino before any software verification. The use of LLMs (ChatGPT, Claude, Copilot, etc.) for generating Arduino code or lab report content is prohibited under the Duke Community Standard. Code must be written during lab time or documented in a lab notebook.