# Lab 05: LTI System Properties & Singularity Functions

## RC/RL Circuit Impulse & Step Responses

ECE 280 — Signals & Systems Laboratory

Duke University, Department of Electrical & Computer Engineering

Lab Duration: 2 hours  |  Completion Date: _________________

Student Names: _________________________________     _________________________________

Bench Station: _________________

# Learning Objectives

By the end of this lab, you will:

- Understand causal LTI systems and their fundamental properties (causality, linearity, time-invariance).

- Solve first-order differential equations analytically to predict RC and RL circuit behavior in the time domain.

- Approximate singularity functions (Dirac impulse δ(t) and Heaviside step u(t)) using digital signals on the Arduino.

- Design and build RC and RL circuits on a breadboard and interface them with an Arduino for real-time data acquisition.

- Extract empirical time constants (τ) from measured data using MATLAB curve fitting and compare against theoretical predictions.

- Quantify agreement between theory and experiment via percent error analysis.

# Background: LTI Systems, Differential Equations & Singularity Functions

## What is an LTI System?

A Linear Time-Invariant (LTI) system is one that satisfies two key properties:

- Linearity: A·x₁(t) + B·x₂(t) → A·y₁(t) + B·y₂(t) for any constants A, B and inputs x₁, x₂.

- Time-Invariance: A time-shifted input produces an identically time-shifted output: x(t − t₀) → y(t − t₀).

RC and RL circuits are classic first-order LTI systems. Their dynamics are governed by linear ODEs with constant coefficients, making them ideal for studying system response.

## Impulse & Step Response

The complete behavior of any LTI system can be characterized by two fundamental test signals:

- Step Response s(t): Output when the system is suddenly excited with a constant DC voltage (u(t) = 1 for t ≥ 0).

- Impulse Response h(t): Output when the system is excited with a narrow, high-amplitude pulse that approximates the Dirac delta δ(t).

In this lab, you will measure these responses from your RC/RL circuits and fit them to theoretical models.

## Time Constant τ

For a first-order circuit, the response decays (or rises) exponentially with a characteristic rate determined by the time constant τ:

•  RC Circuit: τ = RC (seconds)

•  RL Circuit: τ = L/R (seconds)

At time t = τ, the response has decayed to ~36.8% of its initial value (or risen to ~63.2% of steady-state for a step). This lab will measure τ empirically and compare it to your theoretical calculation.

# Pre-Lab Activity: Theoretical Derivations

This section must be completed before arriving at the lab. Your TA will verify your hand-derived equations and theoretical predictions at the start of lab.

## Component Values

⚠️  CRITICAL: Each bench station has uniquely labeled physical resistor(s), capacitor, and inductor. You must not share your component values digitally with other groups. The TA will hand-inspect and verify your specific components before you proceed.

Record your component values below (measured with multimeter if needed):

| Component | Nominal Value | Measured Value |
| --- | --- | --- |
| Resistor (R) | 10 kΩ (typical) |  |
| Capacitor (C) | 100 nF (typical) |  |
| Inductor (L) | 10 mH (typical) |  |

## Part A: RC Low-Pass Circuit Derivation

Consider an RC low-pass circuit with a resistor in series with a capacitor. The capacitor voltage v_C(t) is the output, and a digital input signal x(t) is applied across the RC pair.

✓ Step 1: Write Kirchhoff's voltage law (KVL) around the RC loop and derive the first-order ODE for v_C(t):

✓ Step 2: Solve the ODE for the step response (assume x(t) = V_in for t ≥ 0, and v_C(0⁻) = 0):

Theoretical Step Response: v_C(t) = [your solution here]

✓ Step 3: Compute your theoretical time constant τ_RC = RC (using your measured component values):

τ_RC (theoretical) = __________ μs  or  __________ ms

✓ Step 4: Briefly predict what the impulse response h(t) should look like (e.g., h(t) = (1/τ) exp(−t/τ) u(t) or similar):

## Part B: RL Low-Pass Circuit Derivation

Now consider an RL low-pass circuit with a resistor in series with an inductor. The resistor voltage v_R(t) (or inductor current i_L(t)) is the output.

✓ Step 1: Write KVL for the RL circuit and derive the first-order ODE (choose output as current or voltage across R):

✓ Step 2: Solve for the step response (assume x(t) = V_in for t ≥ 0, and i_L(0⁻) = 0):

Theoretical Step Response: y(t) = [your solution here]

✓ Step 3: Compute your theoretical time constant τ_RL = L/R:

τ_RL (theoretical) = __________ μs  or  __________ ms

✓ Step 4: Predict the impulse response h(t) for the RL circuit:

# Hardware Setup: The Physical Circuits

## Circuit Architecture Overview

Your Arduino will control and measure the RC and RL circuits. Here is the signal flow:

•  Arduino Digital Output (e.g., Pin 2): Serves as system input x(t) (0 V or 5 V logic signal)

•  Breadboard RC/RL Circuit: Processes the input and produces output y(t) (analog voltage across C or R)

•  Arduino Analog Input (e.g., Pin A0): Reads back the output voltage y(t) via on-board ADC

•  Serial Link: Arduino streams time-stamped samples to MATLAB for offline curve fitting

## RC Circuit Wiring

The RC circuit is configured as a low-pass filter. Voltage is applied across the series RC combination, and the capacitor voltage is measured.

[INSERT RC CIRCUIT SCHEMATIC HERE — Show: Arduino Digital Output Pin (e.g., Pin 2) → Resistor R → Capacitor C (with other end to GND). Analog input Pin A0 measures voltage across C. Include labels for R, C, GND, +5V reference.]

### RC Wiring Checklist:

- Digital Output pin (e.g., Pin 2) → one end of resistor R

- Other end of R → one end of capacitor C

- Other end of C → GND (ground rail of breadboard)

- Analog input pin (e.g., Pin A0) → junction between R and C (capacitor voltage)

- GND pin from Arduino → ground rail of breadboard

## RL Circuit Wiring

The RL circuit is also a low-pass filter. Voltage is applied across the series RL combination, and the resistor voltage (proportional to inductor current) is measured.

[INSERT RL CIRCUIT SCHEMATIC HERE — Show: Arduino Digital Output Pin (e.g., Pin 3) → Inductor L → Resistor R (with other end to GND). Analog input Pin A1 measures voltage across R. Include labels for L, R, GND, +5V reference.]

### RL Wiring Checklist:

- Digital Output pin (e.g., Pin 3) → one end of inductor L

- Other end of L → one end of resistor R

- Other end of R → GND (ground rail of breadboard)

- Analog input pin (e.g., Pin A1) → junction between L and R (resistor voltage)

- GND pin from Arduino → ground rail of breadboard

⚠️ Safety Reminder: Inductors can produce high-voltage spikes when power is suddenly removed (due to dI/dt effects). Use a flyback diode (1N4148 or similar) in parallel with the RL series combination to protect the Arduino. Ask your TA if you are unsure.

# Software Implementation: Arduino

## Skeleton Code & High-Speed Sampling

Your TA will provide the FastADC_Stream.ino skeleton code. This pre-written code handles:

- High-speed timer interrupt setup for precise sampling (e.g., 10 kHz or higher)

- ADC configuration to read analog inputs in the interrupt

- Serial communication to stream time-stamped data to your computer

- Buffer management to avoid overflow and data loss

You are NOT expected to write the serial communication code from scratch. This avoids frustrating debugging of baud rates, data frames, and buffer corruption. Instead, you will focus on the LTI concepts and curve fitting.

## Approximating Singularity Functions

In practice, the Arduino cannot generate a true Dirac impulse δ(t) or a perfect Heaviside step. Instead, you will use digital GPIO to approximate them:

### Step Function Approximation

Use digitalWrite(PIN, HIGH) to set the output pin to 5 V. The signal will remain HIGH as long as you do not call digitalWrite(PIN, LOW).

Example:

digitalWrite(2, HIGH);  // Start the step response at t = 0
delay(5000);            // Keep HIGH for 5 seconds
digitalWrite(2, LOW);   // Return to 0 V

### Impulse Function Approximation

An impulse δ(t) is approximated by a very narrow pulse. Use a brief HIGH pulse (duration ~5 ms, which is much smaller than your circuit's time constant τ):

Example:

digitalWrite(3, HIGH);   // Start impulse
delayMicroseconds(5000); // 5 ms pulse width
digitalWrite(3, LOW);    // End impulse
delay(5000);             // Wait for response to settle

✓ Your Task: Modify the skeleton code to include two test functions:

- testStepResponse() — Generates a step input and logs ADC data for ~10 τ values.

- testImpulseResponse() — Generates a narrow pulse and logs ADC data.

Your TA will guide you on where and how to insert these function calls into the main loop.

# Software Implementation: MATLAB Data Analysis & Curve Fitting

## Data Acquisition

Your TA will also provide the ReadArduinoSerial.m MATLAB script. This script:

- Opens the serial port connected to the Arduino

- Reads the incoming time-stamped samples

- Parses and stores them into a MATLAB array (time, voltage)

- Returns the data for plotting and curve fitting

✓ Your Task: Run the provided serial reader to acquire data from your Arduino. You may need to specify the COM port (e.g., 'COM3' on Windows, '/dev/ttyUSB0' on Linux) and baud rate (default: 115200). The TA will help you identify the correct port.

## Task 1: Overlay Plot (Measured vs. Theoretical)

Create a plot that overlays your measured circuit response with your theoretical prediction from the pre-lab.

Steps:

- Plot the measured data (time on x-axis, voltage on y-axis) as a line or scatter plot.

- On the same axes, plot the theoretical exponential response using your pre-lab τ and steady-state voltage.

- Add labels: Title, x-label (Time [s]), y-label (Voltage [V]), legend (Measured, Theoretical).

- Save the plot as a PNG or PDF for your final submission.

[INSERT EXAMPLE MATLAB OVERLAY PLOT HERE — Show: x-axis (Time), y-axis (Voltage), blue line for measured noisy data, red dashed line for theoretical exponential. Title: 'RC Circuit Step Response — Measured vs. Theoretical'. Annotate τ on the plot.]

## Task 2: Empirical Time Constant Extraction via Curve Fitting

Extract the measured time constant τ from your noisy data. There are several approaches; choose one:

### Option A: MATLAB fit() Function (Recommended)

MATLAB's fit() or fittype() can fit an exponential model directly to your data:

% Define the exponential model: y = A * exp(-t/tau)
model = fittype('A*exp(-t/tau)', 'independent', 't', 'dependent', 'y');
% Fit to measured data
[fitObj, goodness] = fit(time(:), voltage(:), model, 'StartPoint', [voltage(1), tau_theoretical]);
% Extract fitted tau
tau_measured = fitObj.tau;
A_measured = fitObj.A;
fprintf('Measured tau: %e seconds\n', tau_measured);

### Option B: Log-Linear Regression

For a decaying exponential y(t) = A exp(−t/τ), take the natural log:

ln(y) = ln(A) − t/τ

This is linear: ln(y) = [ln(A)] + [−1/τ]·t. Use polyfit() to fit a line and extract τ:

% Remove zero or negative values
valid_idx = voltage > 0;
time_valid = time(valid_idx);
voltage_valid = voltage(valid_idx);
% Linear fit to log data
p = polyfit(time_valid, log(voltage_valid), 1);
% p(1) is the slope = -1/tau
slope = p(1);
tau_measured = -1 / slope;
fprintf('Measured tau: %e seconds\n', tau_measured);

Both methods are valid. Option B is simpler if you prefer linear regression.

## Task 3: Percent Error Calculation

Compute the percent error between your theoretical and measured time constants:

Percent Error = |τ_theoretical − τ_measured| / τ_theoretical × 100%

τ_RC (theoretical) = __________ μs

τ_RC (measured) = __________ μs

τ_RC (percent error) = __________  %

τ_RL (theoretical) = __________ μs

τ_RL (measured) = __________ μs

τ_RL (percent error) = __________  %

✓ Interpretation: A percent error < 10–15% is excellent. Larger errors may indicate component tolerances, parasitic resistances, measurement noise, or improper circuit construction. Discuss any outliers with your TA.

# Deliverables & TA Sign-Off Checklist

Your lab is complete when you have successfully demonstrated the three checkpoints below. Your TA will initial and date each checkpoint. You must show all work and answer all questions to receive credit.

## Checkpoint 1: Pre-Lab Verification & Component Check

Before you wire anything on the breadboard, the TA must verify your pre-lab work and physical components.

- ✓ Hand-written derivations of RC and RL differential equations (shown to TA)

- ✓ Theoretical time constants τ_RC and τ_RL computed and recorded

- ✓ Physical resistor, capacitor, and inductor identified and measured

- ✓ Component values match nominal (within expected tolerance) or discrepancies noted

TA Name (print): _____________________________  TA Signature: ___________________  Date: __________

## Checkpoint 2: Hardware & Arduino Testing

Show the TA a working Arduino setup that demonstrates both impulse and step responses.

- ✓ RC circuit breadboard construction verified (correct wiring, no loose connections)

- ✓ RL circuit breadboard construction verified (correct wiring, flyback diode installed if needed)

- ✓ Arduino code compiles and uploads without errors

- ✓ Step response test runs: Arduino outputs stable HIGH, captures ADC decay

- ✓ Impulse response test runs: Arduino outputs narrow pulse (~5 ms), captures ringing/response

- ✓ Serial data streams to MATLAB without corruption (use provided script)

TA Name (print): _____________________________  TA Signature: ___________________  Date: __________

## Checkpoint 3: MATLAB Analysis & Final Plots

Submit completed MATLAB analysis with all plots and percent error calculations.

- ✓ Overlay plot (RC): Measured data vs. theoretical exponential (τ_RC marked)

- ✓ Overlay plot (RL): Measured data vs. theoretical exponential (τ_RL marked)

- ✓ Curve fit results: τ_measured (RC and RL) displayed on plots or in output

- ✓ Percent error (RC): |τ_th − τ_meas| / τ_th × 100% = __________ %

- ✓ Percent error (RL): |τ_th − τ_meas| / τ_th × 100% = __________ %

- ✓ Brief written explanation of any discrepancies (noise, tolerances, parasitic effects, etc.)

TA Name (print): _____________________________  TA Signature: ___________________  Date: __________

## Lab Completion Summary

All checkpoints complete?

- Checkpoint 1: ☐ Yes   ☐ No

- Checkpoint 2: ☐ Yes   ☐ No

- Checkpoint 3: ☐ Yes   ☐ No

Instructor/TA Comments:

# Appendix: Quick Reference

## Key Equations

RC Low-Pass:

τ = RC   |   s(t) = V_in(1 − e^(−t/RC)) u(t)   |   h(t) = (V_in/RC) e^(−t/RC) u(t)

RL Low-Pass:

τ = L/R   |   s(t) = (V_in/R)(1 − e^(−Rt/L)) u(t)   |   h(t) = (V_in/L) e^(−Rt/L) u(t)

## Troubleshooting Guide

- No ADC readings? Check that the Analog input pin is connected to the correct junction (C for RC, R for RL) and that GND is common.

- Voltage stuck at 0 V or 5 V? Verify that your circuit is not shorted or open. Confirm the resistor and capacitor/inductor are properly seated on the breadboard.

- Measured τ very different from theoretical? Check component tolerances (resistors are typically ±5%), possible parasitic inductance in the RC wiring, or Arduino ADC quantization (10-bit, 5 V ref ≈ 4.9 mV per step).

- Serial communication fails? Verify the COM port in MATLAB matches the Arduino IDE device manager. Try restarting the IDE and replugging the USB cable.

- Impulse response looks strange? Ensure your pulse width (~5 ms) is ≪ τ. If τ is too small (e.g., τ < 100 μs), you may need a narrower pulse or higher ADC sampling rate.

## Useful MATLAB Commands

figure, plot, xlabel, ylabel, legend, hold on/off, grid
fit, fittype, polyfit, findpeaks, exp, log, abs, max, min
semilogy (for log-scale plots), xlim, ylim, title

## Useful Arduino Functions

digitalWrite(pin, HIGH/LOW), digitalRead(pin), analogRead(pin)
delay(ms), delayMicroseconds(us), micros(), millis()
Serial.begin(baud), Serial.print, Serial.println

End of Lab 05 Manual